A Process of Serial Update

- Connect PC serial port to STB RS-232C port by 9pin serial cable.

### RS232C Pin connection

| Pin number | Pin Name | PC 9pin connector |
| --- | --- | --- |
| 1 | Not used |  |
| 2 | RXD | 2 |
| 3 | TXD | 3 |
| 4 | Not used |  |
| 5 | GND | 5 |
| 6 | Not used |  |
| 7 | Not used |  |
| 8 | Not used |  |
| 9 | Not used |  |

| # Cable wiring |  |
| --- | --- |
| Female 9pin conn. for PC | Male 9pin conn. for DGTEC STB |
| 2 | 2 |
| 3 | 3 |
| 5 | 5 |

- Run Hyperterminal program in your PC. Refer to below picture.

- Fill in item name to use this program next time.

- Set Connect using item to serial port( COM1 or COM2 ).

- Adjust a serial port properties. Refer to the table and picture below.

| Bits per second | 115200 |
| --- | --- |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |
| Flow control | None |

- If the setting is correct, below command line will be shown.

- Type “RZ”. Refer to the picture below.

- Choose the “Transfer” in the menu, and click the “Send file” menu.

Send file window will be shown. Browse the update file you want and choose the Zmodem protocol. (Note: the filename may be a different version than below.)

- The picture shows all procedure to update file for DGTEC STB.

- All procedure completes after “E.E” sign.

Important: You must not interrupt the Flash ROM transfer, (approx 3 minutes), a

Series of 6 dots will slowly appear before the E.E sign appears.

- Reboot the DGTEC STB.